# REACCH Objective 2 Flux Tower Measurements

# Level 3&4 Output Readme

# Last update: 11 March 2016

# Background Information:

Flux tower measurements are one component of the REACCH Objective 2 Measurements and Monitoring team activities.  The flux towers make field-integrated continuous measurements of carbon dioxide (CO2), water vapor (H2O), and energy fluxes at several sites across the inland Pacific Northwest.  The current sites and brief descriptions are summarized in Table 1, and a satellite image with their locations is given in Figure 1.  The towers are three-meter aluminum frame structures outfitted with a suite of instrumentation for measuring ambient meteorological and trace gas conditions.  Table 2 summarizes the measurements and lists the instruments used for each.

# Table 1: Flux tower sites and summary of important characteristics

| Site abbreviation | MMTN | CFNT | CFCT | LIND | MSLK |
| --- | --- | --- | --- | --- | --- |
| Site name | Moscow Mountain | Cook Farm No-till | Clark Farm Conventional tillage | Lind | Moses Lake |
| Latitude | 46°45'27.90" | 46°47'1.10" | 46°46'39.40" | 46°59'28.82" | 47°0'37.08" |
| Longitude | -116°56'54.96" | -117°4'39.36" | -117°4'50.52" | -118°35'53.88" | -119°14'53.16" |
| Closest town | Moscow, ID | Pullman, WA | Pullman, WA | Lind, WA | Moses Lake, WA |
| Start of measurements | 7/11/2012 | 8/19/2011 | 6/27/2012 | 10/18/2011 | 6/19/2013 |
| Tillage type | Conventional | No-till | Conventional | Reduced tillage | Conventional |
| Irrigation | No | No | No | No | Center pivot |
| Crop rotation* | SW-SB-Pea-WW | WW-SG-WW-SC | WW-SG-WW-SC | WW-fallow | SW-CC-Potato |

*SW (spring wheat), SB (spring barley), WW (winter wheat), SG (spring garbanzo), SC (spring canola), CC (cover crops, i.e. mustard).

Figure 1: Satellite map showing locations of the flux tower sites. Colors represent different agroecological zones: blue=irrigated; green: crop-fallow; red: continuous cropping

# Table 2: Summary of measurements, instruments used, and frequency

| Information | Measurement | Method/Means | Manufacturer, Serial # | Frequency |
| --- | --- | --- | --- | --- |
| Flux | 3-D wind, sonic temperature | 3D sonic anemometer | Campbell Scientific, CSAT3 | 10Hz, continuous |
|  | CO2&H2O | IRGA | Campbell Scientific, EC150 | 10Hz, continuous |
|  | Temperature | Fast temperature probe | Campbell Scientific, EC150 | 10Hz, continuous |
| Meteorological | Temperature, relative humidity | T/RH probe | Vaisala, HMP155A | 10s, continuous |
|  | Net radiation | Net radiometer | Kipp&Zonen, NR-LITE2 | 10s, continuous |
|  | Precipitation | Rain gauge | Texas Electronics, TE525WS | 10s, continuous |
|  | 2-D wind | Cup and vane anemometer | MetOne, 034B | 10s, continuous |
|  | PAR | Quantum sensor | Licor, LI190-SB | 10s, continuous |
|  | Latitude, longitude, altitude, clock change | GPS | Garmin, GPS16X-HVS | 10s, continuous |
| Soil | Soil heat flux | Soil heat flux Plates | Campbell, HFP01SC | continuous |
|  | Soil temperature and moisture | Probes | Decagon, 5TM | continuous |
|  | Bulk density | Lab | n/a | Month/Seasonal |
|  | Total nitrogen | Lab | TruSpec | Month/Seasonal |
|  | Total carbon | Lab | TruSpec | Month/Seasonal |
| Crop | In-canopy PAR | Line Quantum Sensor | Licor, LI191 | Month/Seasonal |
|  | Canopy height | Tape Measure | n/a | Month/Seasonal |
|  | Biomass | Lab | n/a | Month/Seasonal |
|  | Total carbon | Lab | TruSpec | Month/Seasonal |
|  | Total nitrogen | Lab | TruSpec | Month/Seasonal |
| Activity | Tilling | Logbook/e-mail/virtual log | n/a | Seasonal |
|  | Seeding |  | n/a | Seasonal |
|  | Fertilization |  | n/a | Seasonal |
|  | Harvest |  | n/a | Seasonal |
|  | Installation |  | n/a | As needed |
|  | Sampling |  | n/a | As needed |
| Management | Crop rotation | Correspondence with Producer | n/a | Seasonal/Annual |
|  | Fertilization rate |  | n/a | Seasonal/Annual |
|  | Tillage practice |  | n/a | Seasonal/Annual |
| EddyPro Metadata | Altitude | GPS | Garmin, GPS16X-HVS | One-time |
|  | Canopy height | Tape Measure | n/a | Seasonal |
|  | Latitude | GPS | Garmin, GPS16X-HVS | One-time |
|  | Longitude | GPS | Garmin, GPS16X-HVS | One-time |
|  | Height of sonic | Tape Measure | n/a | As needed |
|  | Height of IRGA | Tape Measure | n/a | As needed |
|  | Separation of IRGA (N, E, Vertical) | Tape Measure | n/a | As needed |
|  | Height of inlet tube | Tape Measure | n/a | As needed |
|  | Tube length | Tape Measure | n/a | As needed |
|  | Tube inner diameter | Factory | n/a | As needed |
|  | Nominal flow rate | Flowmeter | n/a | As needed |

# Data Processing Steps:

The data are recorded by a CR3000 data logger (Campbell Scientific), into several different files; the two that are utilized in data processing are “tsdata”, which contains a 10Hz time series of the data that are used in flux calculations, and “stats30”, which contains 30-minute averages of meteorological and auxiliary variables, as well as the Campbell Scientific program’s calculated fluxes for CO2, latent heat, and sensible heat.  All of the files are saved to a compact flash (CF) card, which is exchanged every three weeks.  In addition, the stats30 files are sent to the flux tower group’s database via a cellular modem each night.  During the growing season, measurements of canopy height and biomass are taken.  The biomass is measured destructively: we harvest a square meter, dry it, weight it, grind it, then analyze it for carbon and nitrogen content.  All of the biomass info, in addition to management activities such as planting, fertilizing, tilling, spraying with herbicide, and harvesting are recorded in the site logbook.  The information is also recorded electronically in Ameriflux Biologic, Ancillary, and Disturbance Metadata (BADM) template files for each site.

A flow diagram of the data processing steps is depicted in Figure 2.  All information originates from either the CR3000 data files on the CF card or from observations/measurements recorded in the logbook, as discussed above.  These sources are color-coded green in Figure 2.  From here, there are several steps to get to usable flux and auxiliary data. Hence there are multiple “levels” of data:

- L0: raw 10 Hz and 30-minute average data in binary format; no processing has been done to these data files.

- L1:

  - 10 Hz data segregated into series of half-hour or daily*.dat files (ASCII), ready to be imported into EddyPro

  - 30-min stats30 files in ASCII

- L2: 10 Hz and 30 min average data that are the input for the QA/QC and filtering process.

  - EddyPro output (processed fluxes using the 10 Hz L1 data files)

  - Stats30 processed: monthly files of 30-minute average flux and meteorological parameters, processed into the proper format

- L3: flux and slow data that have passed through the Igor/MatLab quality assessment/quality control (QA/QC) procedure (see below).  These data files will be archived in the REACCH database.

- L4: same as L3 data, but gap-filled.

Levels 0-2 (blue) are not archived on the REACCH database, they will be saved on an ftp site.  Levels 3 and 4 (purple) are the final product for flux measurements.

Figure 2: Data flow diagram showing the steps between raw data collection and final archivable flux data.

# L3 Data Description:

L3 data can be broken into two types depending on the time resolution: 30-minute and daily data.  Both file types contain months’ worth of data; the time frame is indicated by the file name.  Table 2 lists and describes all of the parameters in the L3 30-minute data output.  Table 3 does the same, but for the daily outputs.  Table 2 also includes the source of the parameter, either:

- Campbell: parameters that are taken directly from the CR3000 Campbell Scientific data logger, requiring no calculation or QA/QC; mostly meteorological/auxiliary parameters.

- EddyPro: parameters that were computed/ determined in EddyPro but not QA/QC’ed.

MatLab: parameters that were either QA/QC’ed, filtered, or computed/determined as part of the MatLab post-processing procedure from the MatLab QA/QC and filtering procedures.  All of the daily output parameters were calculated/determined in MatLab.

A few of the 30-minute parameters require further explanation than what is offered in the table:

- CO2qcFlag, H2OqcFlag, HqcFlag, and LEqcFlag: these flags are bit-maps, wherein each digit of the flag indicates whether the given half hour period of a given flux violated one of the QAQC parameters. There are eight flags in all, going up to the 10,000,000 place in increasing order:

  - 1: EddyPro qc flag of “2” for lowest quality data, based on the steady state test and turbulent conditions test

  - 20: Battery voltage <10.6

  - 300: Precipitation in the last 2 hours

  - 4000: u* (u_star)>0.13, indicating low turbulence

  - 50000: winds from behind the tower

  - 600000: IRGA or sonic uptime ratio <80%

  - 7000000: failed “soft-outlier” test (point >3 standard deviations greater or less than the average of the surrounding month)

  - 80000000: no EddyPro output for the flux (data loss due to instrument failure, low power, etc)

The reason the flags are “graduated” in tens places is because a single period could fail multiple QA/QC tests.  For example, a flag of 654300 would indicate that during that half hour, the IRGA uptime ratio was <80%, the winds were blowing from behind the tower, u* was too low, and it had rained in the last two hours.  The reason the flags are not in binary (1’s or 0’s) is for ease of recognition.

- There are five soil temperature and moisture probes deployed at each site: two at 30cm, two at 18cm, and one at 5cm. The order of each is different for each site.

# Table 2: L3 30-minute file description

| Parameter Name | Units | Description | Source |
| --- | --- | --- | --- |
| DateTime | [mm/dd/yyyy HH:MM] | date and time of end of measurement period | EddyPro |
| DOY | [ddd.ddd] | fractional Julian day | EddyPro |
| CO2Flux | [umolm-2s-1] | corrected and filtered CO2 flux | MatLab |
| CO2qcFlag | [#] | bit-map flag for qc filters (see readme) | MatLab |
| H2OFlux | [mmolm-2s-1] | corrected and filtered H2O Flux | MatLab |
| H2OqcFlag | [#] | bit-map flag for qc filters (see readme) | MatLab |
| H | [Wm-2] | corrected and filtered sensible heat flux | MatLab |
| HqcFlag | [#] | bit-map flag for qc filters (see readme) | MatLab |
| LE | [Wm-2] | corrected and filtered latent heat flux | MatLab |
| LEqcFlag | [#] | bit-map flag for qc filters (see readme) | MatLab |
| CO2MolarDensity | [mmolm-3] | molar density of CO2, calculated from measured mass density | EddyPro |
| CO2MixingRatio | [umolmol-1] | mixing ratio of CO2, calculated from measured mass density, and ambient temperature and pressure | EddyPro |
| H2OMolarDensity | [mmolm-3] | molar density of H2O, calculated from measured mass density | EddyPro |
| H2OMixingRatio | [mmolmol-1] | mixing ratio of H2O, calculated from measured mass density, and ambient temperature and pressure | EddyPro |
| AirTemperature | [K] | mean temp of ambient air, calculated from EC150 auxiliary fast temperature sensor | EddyPro |
| AirPressure | [Pa] | mean pressure of ambient air, calculated from EC 150 auxiliary barometer | EddyPro |
| AirDensity | [kgm-3] | density of ambient air | EddyPro |
| VaporDensity | [kgm-3] | ambient mass density of water vapor, calculated from IRGA H2O | EddyPro |
| SpecifiHumidity | [kgkg-1] | ambient specific humidity on a mass basis | EddyPro |
| DewPointTemperature | [K] | ambient due point temperature | EddyPro |
| WindSpeed | [ms-1] | mean wind speed, calculated from sonic anemometer measurements | EddyPro |
| MaxWindSpeed | [ms-1] | max wind speed, from sonic | EddyPro |
| WindDirection | [deg] | wind direction, calculated from sonic | EddyPro |
| FrictionVelocity | [ms-1] | friction velocity, calculated from sonic measurements | EddyPro |
| TurbulenceKineticEnergy | [m2s-1] | (TKE), calculated from sonic measurements | EddyPro |
| MoninObukhovLength | [m] | (L), calculated from sonic measurements | EddyPro |
| (z-d)/L | [#] | Monin-Obukov stability parameter, calculated from sonic measurements and canopy height` | EddyPro |
| BowenRatio | [#] | ratio of sensible heat flux to latent heat flux | EddyPro |
| Tstar | [K] | scaling temperature | EddyPro |
| FootprintModel | - | model used for footprint estimation | EddyPro |
| PeakFootprint | [m] | Along-wind distance providing the highest (peak) contribution to turbulent fluxes | EddyPro |
| 90Footprint | [m] | along-wind distance providing 90% (cumulative) contribution to turbulent fluxes | EddyPro |
| CO2Fluxcamp | [mgm-2s-1] | CO2 flux calculated by the Campbell Scientific software | Campbell |
| LEcamp | [Wm-2] | latent heat flux calculated by the Campbell Scientific software | Campbell |
| Hcamp | [Wm-2] | sensible heat flux calculated by the Campbell Scientific software | Campbell |
| ResultWindDirection | [deg] | wind direction measured by cup and vane anemometer | Campbell |
| SonicUptime | [#] | fraction of the time the sonic anemometer was operating properly | Campbell |
| CO2SignalStrength | [#] | signal strength of CO2 detection by the IRGA (out of 1) | Campbell |
| H2OSignalStrength | [#] | signal strength of H2O detection by the IRGA (out of 1) | Campbell |
| AmbientTemperature | [degC] | ambient temperature measured by the HMP probe | Campbell |
| AmbientPressure | [kPa] | ambient pressure measured by pressure probe | Campbell |
| IRAGuptime | [#] | fraction of the time the IRGA was operating properly | Campbell |
| Tair | [degC] | Air temperature measured by HMP probe | Campbell |
| RH | [%] | relative humidity, measured by HMP probe | Campbell |
| VaporPressure | [kPa] | vapor pressure (e) measured by HMP probe | Campbell |
| SaturatedVaporPresure | [kPa] | saturation vapor pressure (es) measured by HMP probe | Campbell |
| VaporPressureDeficit | [kPa] | Vapor pressure deficit, calculated from HMP measurements of vapor pressure and saturation vapor pressure | MatLab |
| NetRadiation | [Wm-2] | incoming minus outgoing long and short- wave radiation, measured by a net radiometer | Campbell |
| PPFD | [umolm-2s-1] | photosynthetic photon flux density, measured by a PAR sensor | Campbell |
| Precipitation | [mm] | cumulative precipitation, measured by a tipping-bucket rain gauge | Campbell |
| SoilDielectricPermittivity#5 | [#] |  | Campbell |
| SoilTemperature#5 | [degC] |  |  |
| SoilMoisture#5 | [#] | derived from dielectric permittivity, units are m3/m3 | Campbell |
| SoilDielectricPermittivity#6 | [#] |  |  |
| SoilTemperature#6 | [degC] |  |  |
| SoilMoisture#6 | [#] |  |  |
| SoilDielectricPermittivity#7 | [#] |  |  |
| SoilTemperature#7 | [degC] |  |  |
| SoilMoisture#7 | [#] |  |  |
| SoilDielectricPermittivity#8 | [#] |  |  |
| SoilTemperature#8 | [degC] |  |  |
| SoilMoisture#8 | [#] |  |  |
| SoilDielectricPermittivity#9 | [#] |  |  |
| SoilTemperature#9 | [degC] |  |  |
| SoilMoisture#9 | [#] |  |  |
| SoilHeatFlux#1 | [Wm-2] |  |  |
| SoilHeatFlux#2 | [Wm-2] |  |  |
| BatteryVolt | [V] | voltage supplied from battery bank |  |

# Table 3: Daily L3 file description

| Parameter Name | Units | Description |
| --- | --- | --- |
| Date | [mm/dd/yyyy] | date |
| PPFD_avg | [umolm-2s-1] | photosynthetic photon flux density, average for the day |
| PPFD_std | [umolm-2s-1] | standard deviation of PPFD |
| PPFD_max | [umolm-2s-1] | maximum PPFD |
| Tair_avg | [degC] | ambient temperature average |
| Tair_std | [degC] | standard deviation of T |
| Tair_max | [degC] | maximum T |
| Tair_min | [degC] | minimum T |
| VPD_avg | [kPa] | average vapor pressure deficit |
| VPD_std | [kPa] | standard deviation of VPD |
| Precipitation | [mm] | total (cumulative) precipitation for the day |
| H_avg | [Wm-2] | average sensible heat flux |
| H_std | [Wm-2] | standard deviation of H |
| H_max | [Wm-2] | maximum H |
| LE_avg | [Wm-2] | average latent heat flux |
| LE_std | [Wm-2] | standard deviation of LE |
| LE_max | [Wm-2] | maximum LE |
| CO2Attr1 | [%] | percent of "good quality" CO2 flux periods, not filtered by EddyPro, QA/QC parameters, or soft outlier statistical filter |
| CO2Attr2 | [%] | percent of CO2 flux periods filtered by QA/QC parameters or soft outlier statistical filter |
| CO2Attr3 | [%] | percent of CO2 flux periods missing because EddyPro could not calculate the flux; gaps due to instrument error, loss of power, etc |
| H2OAttr1 | [%] | same as CO2 attribution labels above, but for H2O fluxes |
| H2OAttr2 | [%] |  |
| H2OAttr3 | [%] |  |
| HAttr1 | [%] | same as CO2 attribution labels above, but for sensible heat fluxes |
| HAttr2 | [%] |  |
| HAttr3 | [%] |  |
| LEAttr1 | [%] | same as CO2 attribution labels above, but for latent heat fluxes |
| LEAttr2 | [%] |  |
| LEAttr3 | [%] |  |

# L4 Data Description:

Similar to L3 data, there are two L4 final datasets: one with data at a 30-minute time resolution and one daily file.  The L4 30-minute gap-filled data is very similar to L3 data, except that the fluxes of CO2, water vapor, and energy have been gap-filled using one of three ways: with the Campbell Scientific flux data, with the mean diurnal variation method (MDV), or with a non-linear regression (NLR).  There are also four additional output parameters (one per flux) called “fluxGapFilling” which indicate if and how the given flux for the given thirty-minute period was gap-filled.  The gap-filling flag will read either:

- 0 (zero): good quality EddyPro output; not gap-filled

- 1: gap-filled using Campbell Scientific fluxes

- 2: gap-filled using MDV

- 3: gap-filled using NLR

The L4 daily output file is also similar to L3, with the addition of integrated CO2 and H2O fluxes now that the data has been gap-filled. The additional parameters include:

- CO2_daily (gCm-2d-1): CO2 flux, or net ecosystem exchange (NEE) integrated over the day

- Reco_daily (gCm-2d-1): total ecosystem respiration (plant and soil) integrated over the day

- GPP_daily (gCm-2d-1): gross primary productivity integrated over the day

- H2O_daily (mmd-1): H2O flux integrated over the day

# Jargon glossary:

- Eddy covariance method: a technique for measuring the net emission or uptake of a scalar quantity (a trace gas or energy).  By making fast measurements of the vertical wind speed and trace gas concentration, we can determine the concentration in updrafts and downdrafts, and by averaging how the wind and concentration co-vary over time, determine the flux.

- EddyPro: a flux processing software developed and maintained by the company LiCor Biosciences.  It is based on the software ECO2S, developed by a flux group at the University of Tuscia in Italy (http://www.licor.com/env/products/eddy_covariance/software.html).

- Flux: flow per unit area. In the case of CO2, we measure umol/m2/s.

- Flux tower: shorthand for the suite of instrumentation, mounted on a three-meter aluminum frame tower, used to measure ambient conditions necessary for calculating fluxes of CO2, water, and energy.  In addition to what is mounted on the tower, each site also has two 70-watt solar panels and a battery bank of six deep cycle batteries.

- Flux tower site: refers to one of the five locations where flux towers have been installed as a part of the REACCH project, and also includes the field or fetch that is being measured by the tower.

- Hertz (Hz): unit of 1/s, so 10Hz means ten measurements per second.